Studies on a modern method of preparing  authentic mauveine

M. John Plater* and Andrea Raab Department of Chemistry, University of Aberdeen, Meston Walk, Aberdeen, AB24 3UE, e-mail: m.j.plater@abdn.ac.uk

Graphical abstract

Mauveine made by different methods is analysed by HPLC and compared with authentic mauveine. These results cast doubt on the accuracy of the historical method used to make mauveine by Perkin.

Key words: HPLC, tertiary butanol, trimethylcarbinol, mauveine, mauve

Abstract

The composition  of modern mauveine I and II is analysed by HPLC and compared with authentic mauveine and a standard consisting of mauveine A, B2, B and C. The composition of modern mauveine II is similar to that of authentic mauveine.

Introduction

Reading the two volumes1-2 of the British patent index Bleaching, Dyeing and Printing Calico, 1617-1876, and Bleaching, Dyeing and Printing Calico and other Fabrics, 1877-1883,3  gives an insight into the historical changes that occurred with the discovery of the coal-tar dyes. Prior to 1856, the year in which Perkin patented mauveine,4 dyestuffs were usually prepared from plant, animal or mineral sources. From 1856 many patents appear on dyes made by chemical synthesis, for which mauveine was the first to be commercialised. This patent marks the beginning of the coal-tar dye industry which spread rapidly through Britain, France, Germany and Switzerland.5 It has had numerous celebrations and reviews.6-10 Luckily there are some samples of mauveine remaining which have been analysed and their composition has been determined.11  Both Schunck’s mauveine, in the Museum of Science and Industry (MOSI) Manchester, and Caro’s mauveine in the Deutsche Museum, are the same and are both rich in pseudo-mauveine.12 These were probably prepared by Dale and Caro’s method using CuCl2 as oxidant.10 Mauveine samples from the Science Museum in London, MOSI in Manchester and the Chandler Museum in Arizona are similar and are all rich in mauveine A and B.12 MOSI mauveine, from Blakely,13 is sealed with a Perkin & Sons cork so most likely came from Perkin’s factory. Our studies have centered upon the preparation of derivatives similar to those found in mauveine,14-15 and upon the synthesis of mauveine which has the same composition as authentic mauveine.16-17 This is because mauveine prepared by the potassium dichromate oxidation of a mixture of p-toluidine, o-toluidine and aniline, a method we dubbed Perkin’s stated method, gives a standard product mixture of mauveine A, B2, B and C.18 These four products were isolated and fully characterised.18 However, authentic mauveine is rich in mauveine A and B only. Although a comparison of low resolution HPLC traces of authentic mauveine vs as synthesised mauveine was made, we felt that the significance was overlooked and the trace was of low resolution.18 We published photographs of TLC plates to try and give a visual indication of the experimentation since the chromophores are coloured.16-17 So far in our hands we cannot make authentic mauveine by following Perkin’s method.  However, we were able to get a much better match to authentic mauveine by the potassium dichromate oxidation of tert-butyl-p-toluidine, o-toluidine and aniline followed by the deprotection of a mixture of intermediate tert-butylated chromophores. So p-toluidine has been replaced by tert-butyl-p-toluidine which increases the yield considerably. The preference for mauveine A and B was rationalised by assuming that  tert-butyl-p-toluidine could no longer couple to give mauveine B2 or C. Aniline would always couple into the center position. This is easy to see from the drawings below in figure 1.

Figure 1 Drawings of mauveine A, B2, B and C. The reader should compare A with B2, and B with C to see where aniline has been replaced with p-toluidine.

This paper reports our HPLC studies on mauveine made using tert-butyl-p-toluidine and makes comparisons to authentic mauveine.

Discussion

HPLC traces are shown in figures 2-5 and some ratios of peak integrations are in table 1. A number of spiking studies were performed to allow accurate identification of compounds with the same retention times (see experimental section). Chart 1 is of authentic mauveine. Mauveine A and B have previously been characterised spectroscopically by others13 and by us from our synthetic work.16-17 We also separated small quantities of mauveine A and B from authentic mauveine and confirmed the molecular ions to be correct. The predominance of mauveine A and B is clear, which are approximately equal, and the extra fingerprint peak Bx,  after mauveine A,  is about 30% of the mauveine A peak.  Chart 2 shows a standard of  mauveine A, B2, B and C. The standard was made as described by de Melo and all four compounds have previously been isolated and characterised spectroscopically.18 Chart 3 is a mixture of the standard and authentic  mauveine. The A and B peaks align but the so called fingerprint peak Bx in authentic mauveine is not mauveine B2. Likewise the chromophore to the right of mauveine B, in authentic mauveine, is not mauveine C. We call this peak Cx. The two samples, of approximately equal concentration, must be mixed in a vial prior to injection as the retention times are quite close. The Bx and Cx peaks, in authentic mauveine, have  unknown identity’s. Since they are minor components of the mixture, and authentic mauveine is in short supply, they will probably be hard to characterise. These two peaks, Bx and Cx, were previously suggested as being mauveine B2 and C by comparison to the standard, but the HPLC trace published was of low resolution.18 Our samples sometimes show a Cx peak immediately after mauveine B, but not always, depending upon the composition of the aryl amines. Chart 4 shows a typical mixture of chromophores, mauveine I, prepared using tert-butyl-p-toluidine, which we photographed previously. Chart 5 shows this mixture spiked with the standard and chart 6 shows this mixture spiked with authentic mauveine. Again the fingerprint peak Bx is not mauveine B2 and has the same retention time as the compound Bx in authentic mauveine, chart 6. The C group chromophores are just visible in chart 4. These observations suggests that modern mauveine I might be made the same way as authentic mauveine, and by a method which is different from that of the standard, Perkin’s stated method. These studies show that the ratio of mauveine A/B is still to low compared to that in authentic mauveine, and the amount of the fingerprint component Bx is to high compared to mauveine A (table 1). After some experimentation it was discovered that increasing the quantity of acid favours more mauveine A, and that the ratio of aniline to o-toluidine can be reduced at lower pH (chart 7 of mauveine II). This actually gives a good  ratio of mauveine A/mauveine Bx (fingerprint peak)  and a good match for the mauveine A/B ratio compared to authentic mauveine (table 1). Also a moderate increase in acid increases the yield from 4-5% to about 8-9% whereas the yield of mauveine by Perkin’s method is about 1-2% in our hands. Chart 8 shows the standard with this chromophore mixture and chart 9 shows authentic mauveine with this mixture. In both cases the solutions were mixed in a sample vial prior to injection. The mixture with the standard shows that the fingerprint peak Bx, of modern mauveine II, is different from peak B2 of the standard, as expected. The mixture with authentic mauveine verify’s that peaks A, Bx and B of modern mauveine II have the same retention time as the corresponding peaks in authentic mauveine, as before for chart 6.

Figure 2 HPLC charts 1-3 of mauveine samples or spiked samples.

Chart 1: Authentic mauveine.

Chart 2: Standard of mauveine A, B2, B and C.

Chart 3: Standard mixed with authentic mauveine.

Figure 3 HPLC charts 4-6 of modern mauveine sample I or spiked samples.

Chart 4: Modern mauveine I. See experimental section for preparation using K2Cr2O7

Chart 5: Standard of mauveine A, B2, B and C mixed with modern mauveine I.

Chart 6: Authentic mauveine mixed with modern mauveine I.

Figure 4 HPLC charts 7-9 of modern  mauveine II or spiked samples.

Chart 7: Modern mauveine II. See experimental section for preparation using K2Cr2O7.

Chart 8: Standard of mauveine A, B2, B and C mixed with modern mauveine II.

Chart 9: Authentic mauveine mixed with modern mauveine II.

Figure 5 HPLC chart 10 of Schunck’s mauveine.

|  | Sample | A | Bx | B | A/Bx | A/B |
| --- | --- | --- | --- | --- | --- | --- |
| Chart 2 | Authentic mauveine | 29 | 10 | 34 | 2.9 | 0.85 |
| Chart 4 | Mauveine I | 13 | 14 | 47 | 0.93 | 0.28 |
| Chart 7 | Mauveine II | 27 | 10 | 31 | 2.7 | 0.87 |

Table 1 Integral peak areas and rati’s from authentic  mauveine and mauveine made using tert-butyl-p-toluidine. The concentrations are not standardised for different samples.  Chart 4: Mauveine I made by oxidising (i) tert-butyl-p-toluidine, aniline and o-toluidine (1.0:3.0:1.5) in aqueous acid (0.30-0.45 ml cH2SO4 in 100-200 ml H2O); Chart 7: (ii) Mauveine II made by oxidising (i)   tert-butyl-p-toluidine, aniline and o-toluidine (1.0: 1.5: 1.5) in aqueous acid (0.6 ml cH2SO4 in 100 ml H2O).

Schuncks mauveine and Caro’s mauveine are the same and are rich in pseudomauveine.11-12 Chart 10 is of Schunck’s mauveine. Aniline itself was also oxidised with CuCl2 . 2H2O which gave just pseudo-mauveine. In the Dale and Caro patent only aniline was oxidised,19 which had not been enriched with toluidines, so this might explain the formation of mauveine rich in pseudo-mauveine by this method. The purity of the amines will affect the ratio of the products which form.

Summary

HPLC studies show that mauveine can be made, using tert-butyl-p-toluidine, which is a good match to the composition of authentic mauveine. HPLC studies also show that the fingerprint peak Bx in authentic mauveine, 30% of mauveine A in intensity, has the same retention time as the corresponding fingerprint peak in modern mauveine I and II, made from tert-butyl-p-toluidine. These three identical peaks (Bx) were shown to have a different retention time to the corresponding peak in the standard (B2) made by Perkin’s method. Peak Cx in authentic mauveine is also different from mauveine C in the standard made by Perkin’s method. Even without knowing what  these fingerprint peaks are, these differences in retention time, and  the different compositions, suggests the possibility of a different or modified manufacturing process than the popular notation.

A chemical inventory for Perkin’s factory, contained in a lawsuit filed by the new owners in 1874, is in the supplementary information. This gives an insight into the chemicals that might have been available to make coal-tar dyes. The current results cast some doubt on the accuracy of the historical description of the process used to make mauveine by Perkin.

Experimental

General: IR spectra were recorded on an ATI Mattson FTIR spectrometer using KBr discs. UV spectra were recorded using a Perkin-Elmer Lambda 25 UV-VIS spectrometer with CH2Cl2 as the solvent. 1H and 13C NMR spectra were recorded at 400 MHz and 100.5 MHz, respectively, using a Varian 400 spectrometer. Chemical shifts, δ are given in ppm relative to the residual solvent and coupling constants, J are given in Hz. Low resolution and high resolution mass spectra were obtained at the University of Wales, Swansea using electron impact ionisation and chemical ionisation. Melting points were determined on a Kofler hot-stage microscope. HPLC used an Agilent 1200 Infinity Series 1220 LC System VL with a 15 cm  long C18 reverse phase column and gradient elution using H2O/0.1% formic acid/MeOH over 45 min [10 min, 50/50; 20 min, 40/60; 30 min, 25/75; 35 min, 0/100; 40 min, 50/50].  The detector was set at 550 nm. Mauveine I and II and all other samples of mauveine were purified by two chromatography columns prior to HPLC analysis. Minor differences in purity in the HPLC charts, compared to authentic mauveine, probably reflects the differences in method of purification.

Spiked samples are approximately 1:1 sample concentrations of 0.2 mg/ml of each component. The individual sample solutions that were mixed (0.75 ml of each) were the same solutions used for the single analyses.

Authentic mauveine from MOSI (HPLC chart 1)

Standard consisting of mauveine A, B2, B and C. (HPLC chart2) This method of synthesis has been reported previously.18

Authentic mauveine prepared from tert-butyl-p-toluidine

Method 1 (Modern mauveine I) See previous paper and photograph of the TLC plate (HPLC chart 4).17

Method 2 (Modern mauveine II)  (HPLC chart 7)

N-tert-Butyl-p-toluidine hydrochloride (373 mg, 1.87 mmol, 1.0 eq), o-toluidine (300 mg, 2.8 mmol, 1.5 eq) and aniline (261 mg, 2.8 mmol, 1.5 eq) were dissolved in distilled water (100 ml) acidified with cH2SO4  (12 drops, 0.6 ml ) then treated with K2Cr2O7 (0.95 g, 3.20 mmol) and stirred at 40-45 0C for 12 h. The purple reaction mixture was filtered through a fine pore sinter and washed with cold water (200 ml). The precipitate was extracted with MeOH (40 ml x 8) in the sinter each time agitating the precipitate in MeOH before filtration through the sinter under vacuum. The combined extracts were heated to dryness. The product was purified by chromatography on silica gel. MeOH firstly eluted impurities then cNH3/MeOH (20:80) eluted a mixture of purple chromophores that were not separated. After evaporating to dryness in a beaker the product was then treated with MeOH (10 ml) then cHCl (10 ml) and evaporated to dryness on a hotplate with a fume hood on. The product was purified by chromatography on silica gel. MeOH firstly eluted a small amount of impurity then cNH3/MeOH (20:80) eluted predominantly a mixture of mauveine A and mauveine B (75 mg, 8.5%) (85 % from the above tert-butylated mixture). The sample analysis was verified by purification of a small sample (25 mg) by chromatography on silica gel and collecting fractions. Elution with secBuOH:EtOAc:H2O:HOAc (60:30:9.5:0.5) gave firstly mauveine B then mauveine A with the same spectroscopic properties to literature material.17

Schunck’s mauveine (HPLC chart 10) This was obtained from the Museum of Science and Industry, Manchester.

Pseudomauveine Aniline (500 mg, 5.38 mmol) in H2O (100 ml) and cH2SO4 (6 drops, 0.3 ml) was treated with CuCl2 .2H2O (4.6 g, 27.0 mmol) and stirred at 50 0C for 12 h. After cooling the reaction was filtered, then extracted with MeOH (6 x 50 ml). The combined MeOH extracts were evaporated to dryness and the mixture was purified by chromatography on silica gel. Firstly MeOH eluted front running impurities then cNH3/MeOH (80:20) eluted the title compound (13 mg, 0.6 %) as a dark solid. The product was identical to material reported previously by us.14 The proton and carbon spectra are in the supplementary section.

Supplementary material

NMR spectra for pseudo-mauveine.

New owners chemical inventory of Perkin’s factory 1st Jan 1874.

References

1. Bleaching, Dyeing and Printing Calico, G. E Eyre and W. Spottiswoode, Part I  1617-1857.

2. Bleaching, Dyeing and Printing Calico, G. E Eyre and W. Spottiswoode,  Parts II and III 858-1876.

3. Bleaching, Dyeing and Printing Calico and other Fabrics, G. E Eyre and W. Spottiswoode, 1877-1883.

4. W.H. Perkin, 1856, No 1984, August 26th.

5. W.H. Perkin, The British coal tar industry, its origin, development, and decline, ed. W.M. Gardner. J.B. Lippincott Company and Kessinger Publishing LLC, Philadelphia, pp. 141–187 (Hofmann Memorial Lecture) [modern print]; W. H. Perkin J. Chem. Soc., 1896, 596.

6. A. S. Travis, Bull. Hist. Chem., 2007, 32, 35.

7. W.H. Cliffe, J. Soc. Dyers Colour., 1956, 72, 563.

8. C. J. W. Hooper, J. Soc. Dyers and Colour., 1956, 72, 566.

9. P. J. T. Morris, Hist. and Tech., 2006, 22, 119.

10. I. Holme, Colour. Technol., 2006, 122, 235.

11. M.M. Sousa, M.J. Melo, A.J. Parola, P.J.T. Morris, H.S. Rzepa and J. Sergio Seixas de Melo, Chem. Eur. J., 2008, 14, 8507.

12. M. da Conceicao Oliveira, A. Dias, P. Douglas and J. S. Seixas de Melo, Chem. Eur. J., 2014, 20, 1808.

13. O. Meth-Cohn and M. Smith, J. Chem. Soc., Perkin Trans., I 1994, 5.

14. M.J. Plater, J. Chem. Res., 2011, 35, 304.

15. M.J. Plater, J. Chem. Res., 2013, 37, 427.

16. M.J. Plater, J. Chem. Res., 2014, 38, 163.

17. M. J. Plater, J. Chem. Res., 2014, 38, 651.

18. J. Seixas de Melo, S. Takato, M. Sousa, M.J. Melo and A.J. Parola, Chem. Commun., 2007, 2624.

19. J. Dale and H. Caro, 1860, No 1307, May 26th.

Acknowledgements

We thank the EPSRC National Mass Spectrometry Service Centre (University of Swansea) for the high-resolution mass spectra and the Manchester Museum of Science and Industry (MOSI) for samples of authentic mauveine and Schunck’s mauveine.