Effect of pH adjustment, solid-liquid separation and chitosan adsorption on pollutants’ removal from pot ale wastewaters

Davide Dionisi,* Materials and Chemical Engineering group, School of Engineering, University of Aberdeen, Aberdeen, AB24 3UE, UK

Sarah Sine Bruce, Materials and Chemical Engineering group, School of Engineering, University of Aberdeen, Aberdeen, AB24 3UE, UK

Malcolm John Barraclough, OMB Technology, Claylands Farm, Balfron, G63 0RR, UK

Abstract

Pot ale is a wastewater from the whisky industry which is produced in large volumes and causes significant environmental concern. This study investigates the degree of COD, phosphorus, ammonia and copper removal obtained from pot ale using solid-liquid separation, carried out in the range of pH values 3.4-9.0. This study also investigates the removal of the same pollutants, from the liquid phase after solid-liquid separation, obtained by adsorption on unmodified chitosan, in a range of pH values. By solid-liquid separation, a removal of up to 14% of the COD, 60% of free phosphate, 45% of total phosphorus, 65% of ammonia and >80% of copper was obtained. In general, the highest removal of the pollutants was observed at alkaline pH values. Adsorption with chitosan, at an initial pH of the wastewater equal to 5, allowed only a modest COD removal, up to 10%, and up to 35% removal of free phosphate. When the initial pH of the wastewater was adjusted to 7, no removal of COD and phosphorus was observed with chitosan, while adsorption at more acidic pH values was impossible due to formation of a thick paste with water. Adsorption capacity for COD and phosphorus correlated well with the final pH after chitosan addition, and it was shown to decrease sharply with increasing pH. Overall, this study shows that solid-liquid separation removes a significant fraction of the pollutants in pot ale, while chitosan might only be effective after chemical modifications (e.g. cross-linking) which improve its stability at acidic pH.

* email: davidedionisi@abdn.ac.uk, phone: +44 (0)1224 272814

1. Introduction

Pot ale is a wastewater produced by the whisky industry, as the residual of the distillation process (Figure 1). Pot ale is produced in millions m3 per year and causes serious environmental concerns due to the high levels of COD, BOD, phosphorus and ammonia, as well as the presence of copper [1]. Current processes to dispose or treat pot ale include: direct disposal to the sea, spreading on land as fertiliser, evaporation to produce pot ale syrup and anaerobic digestion [2,3]. However, all these methods for disposal or treatment have their limitations. Direct disposal to the sea is only possible in very limited circumstances, where the location of the distillery allows it. Spreading on land as fertiliser causes concern, due to the possible toxic effects of the pollutants contained in pot ale. Evaporation to produce pot ale syrup is expensive due to the high energy costs and the use of pot ale syrup is limited because it cannot be fed to sheep due to its copper content. Anaerobic digestion has large start-up costs and in general is only economically viable for large distilleries.

Research on pot ale treatment is mainly focussed on biological processes, anaerobic digestion or sequences of anaerobic and aerobic treatment steps [4-6]. Other alternative technologies investigated for the treatment of distillery wastewaters include coagulation-flocculation, adsorption, oxidation processes (Fenton’s oxidation, ozonation and electrochemical oxidation) and membrane processes [7,8]. However, none of these technologies can be considered to be totally satisfactory, and they suffer from the disadvantages of the high requirement of chemicals, large sludge generation and high operating costs [9].

This study has the following aims:

- to investigate the removal of the main pollutants in pot ale, i.e. COD, phosphorus, ammonia and copper, which can be obtained by a simple process of solid-liquid separation. The effect of pH on the degree of pollutants’ removal obtained by solid-liquid separation has been investigated;

- to investigate the removal of the pollutants by an adsorption stage carried out using chitosan.

In this study adsorption, rather than other chemical-physical processes such as coagulation or oxidation, was considered because it is easy to implement at plant scale and does not require a large capital cost. Indeed adsorption can be easily carried out using self-contained units, which can be easily moved in and out of the plant, and does not require the installation of any dedicated facility. This is particularly important in relatively small-size plants such as distilleries. Chitosan was chosen for the adsorption process because it is a cheap renewable material, which has been attracting significant interest for wastewater treatment [10]. Chitosan is produced from the partial deacetylation of chitin, which is produced in large quantities as a waste from the seafood processing industry [11]. Therefore there is increasing interest in extending the commercial uses of chitosan, since a larger market for chitosan would also alleviate the waste disposal problem of the seafood industry. Chitosan has potentially better adsorption properties than cellulose, which is also a natural polymer potentially available from waste biomass, because of the presence of amino and acetamide groups which extend the range of substances which can be adsorbed on this molecule. Chitosan has been shown to remove heavy metals [12], dyes [13], phosphate and nitrate [14], COD from rice mill [15] and from biodiesel wastewaters [16].

To the best of our knowledge, neither process investigated in this research, i.e. solid-liquid separation and adsorption with chitosan, has been investigated so far for pot ale wastewaters.

2. Experimental methods

2.1 Pot ale wastewater and chitosan

A sample from pot ale wastewater was received from a distillery in Scotland and used without pre-treatments.

Chitosan was bought from Sigma-Aldrich, product code 448877, and used without pre-treatments. This chitosan is 75-85% deacetylated and has a molecular weight of 190,000-310,000 Da.

2.2. pH adjustment and solid liquid separation

A 2-litre sample of the pot ale wastewater was placed on a magnetic stirrer at 350 rpm. The following parameters were measured for the unmodified pot ale wastewater, as received from the distillery (“raw” wastewater): pH, Chemical Oxygen demand (COD), Total Suspended Solids (TSS), digested copper, digested phosphorus, and free ammonia.

pH was adjusted using a NaOH solution between the original pH of the wastewater (3.42) and the final value of 9.0. At the original pH and at the pH values of 5.0, 6.0, 7.0, 8.0 and 9.0, solid-liquid separation was carried out using filtration on Whatman GF/C filter paper. Filtered samples for measurement of COD, digested copper, free phosphate and digested phosphorus and free ammonia were taken from the filtrate at each pH value.

2.3 Adsorption experiments

Adsorption experiments were carried out using pot ale after pH adjustment and filtration. The pH of the initial wastewater was adjusted to values of 5.0 and 7.0. The adsorption experiments were conducted in 500 ml glass bottles with 50 ml of filtered sample in each, to which various concentration of chitosan were added. A control experiment was also run with no chitosan added.  The bottles were agitated in an orbital shaker running at 200 rpm at a temperature of 20 °C for 18-20 hours to ensure that equilibrium had been achieved. After adsorption had taken place, each bottle was re-filtered using the Whatman GF/C filter paper to ensure all chitosan was removed. The following parameters were measured and compared with the results from the “control” bottle: COD, free phosphate and digested phosphorus, free ammonia and digested copper. An additional experiment was carried out where the pH of the wastewater was controlled to the value of 6, by adding the appropriate concentration of sulphuric acid after chitosan addition before proceeding with the adsorption experiment. The adsorption capacity was calculated as follows:

where C0 is the initial concentration of the adsorbate (COD or free phosphate), C is its final concentration at the end of the adsorption experiment and m is the concentration of chitosan in the experiment.

2.4 Analytical methods

COD, phosphorus, ammonia and copper were measured using the appropriate Spectroquant Cell test method (Merck Millipore, method number 114555 for COD, 100673 for phosphorus, 114558 for ammonia and 114553 for copper), and the Spectroquant Nova 60 photometer. For the determination of digested phosphorus, samples were digested for 30 min at 120 OC, according to the procedure described in the Spectroquant cell test method. For copper analysis, samples were pretreated as follows: 0.1 ml HNO3/ml sample was added, then the samples were digested at 100 OC for 1 h. After cooling to room temperature, the copper concentration was measured.

3. Results and discussion

The initial composition of the pot ale sample used in this study is reported in Table 1.

3.1 pH adjustment and solid-liquid separation

3.1.1 Base addition and pH increase

Figure 2 shows the increase in pH in pot ale sodium hydroxide. When the pH reaches a value of approximately 5, pH starts to increase more rapidly. The slow increase of pH at the beginning is probably due to the presence of weak acids in pot ale, which have a buffering effect on pH. The presence in pot ale of volatile acids, including, among the others, acetic, propionic and lactic acid, has been reported [1], at a total concentration up to 10 g/l. The maximum pH in our tests was set at the value of 9 because this is the maximum pH limit for the discharge of pot ale effluents for the distillery which provided the pot ale sample.

3.1.2 COD removal

Figure 3 shows the COD profile both in the raw pot ale sample at the initial pH and in filtered samples as a function of pH. It can be observed that the raw value is greater, by approximately 10%, than the filtered value for the same pH, indicating that some proportion of COD is insoluble and bound to the solids. This means that by simply removing the solids from the raw pot ale wastewater a 10% decrease in COD may be achieved. As the pH value is increased, a slight decrease, corresponding to about 4% from the lowest to the highest pH value, in the filtered COD levels is observed. Overall, by removing the solids and increasing the pH a reduction of COD of approximately 14% can be achieved. Our results are in qualitative agreement with the results by Tokuda et al. [17], which observed a 20% removal of COD from untreated pot ale, at the original acidic pH, by sedimentation. COD removal due to solid-liquid separation can be explained by the organic nature of the solids in pot ale, which contain dead yeast cells, proteins, etc [17,18].

3.1.3 Phosphorus removal

Figure 4 shows digested phosphorus and free phosphate profiles in the raw pot ale and in filtered samples in a range of pH values. At the initial pH, total phosphorus in the raw pot ale and in filtered samples was virtually the same, therefore indicating that there is virtually no phosphorus bound to solids in the raw pot ale. This is in agreement with what observed in another study [17], where only 5% decrease in total phosphorus was observed by removing the solids using sedimentation.

The difference between digested phosphorus and free phosphates is due to soluble phosphorus-containing species different than phosphates, e.g. polyphosphates. Both free phosphates and digested phosphorus in the filtered samples showed a marked decrease at pH 9. This is likely due to phosphorus precipitation. Interestingly, the difference between digested phosphorus and free phosphates does not change significantly at pH 9 compared to the other pH values, and this means that probably only free phosphates precipitate, while other forms of soluble phosphorus remain in solution. Overall, by adjusting the pH and separating the solids, a reduction in free phosphate of approximately 60% and in total phosphorus of approximately 45% can be achieved. The most likely forms for phosphorus precipitation in this study are as calcium or magnesium salts.  Calcium is likely to be present in pot ale, due its presence in the starting material, barley, in the yeasts and in the water used for the whisky production process. Satyawali and Balakrishnan [9] report a calcium concentration of 0.8 and 0.2% in cane and beet molasses, respectively. Even though these data are not immediately transferable to pot ale, they give an indication of the likely presence of calcium in pot ale at significant concentrations. Phosphorus may precipitate as different species of calcium phosphates, of which hydroxyapatite Ca5(PO4)3OH is thermodynamically the most stable one [19]. Precipitation as calcium phosphate is favoured by alkaline pH values [20] and this may explain the sharp drop in phosphorus concentration observed at pH 9. The reaction of phosphorus precipitation as hydroxyapatite can be described by the stoichiometry below [20]

(1)

Another possibility is phosphorus precipitation as magnesium salt, i.e. as struvite, NH4MgPO4·6H2O. Mg has been reported to be present in pot ale at a concentration of approx. 0.2% [17]. The effect of pH on struvite precipitation is complex and is dependent on the presence of other ions, however in general struvite precipitation is favoured by alkaline pH values and is described by the stoichiometry below [21]

(2)

Struvite formation from pot ale after anaerobic digestion was observed by Tokuda et al. [17], who observed more than 90% phosphate removal by adding a magnesium salt in a process carried out at pH 8.2-8.4.

3.1.4 Ammonia removal

Figure 5 shows the profile of free ammonia as a function of pH. Ammonia concentration remains virtually unaffected by pH until pH 8, and then it shows a sharp drop at pH 9, with approx. 65% ammonia removal. The profile of ammonia as a function of pH can be explained, similarly as what discussed in a previous section for phosphorus removal, by ammonia precipitation as struvite, described by equation (2), which is favoured by alkaline pH values. Ammonia removal as struvite precipitate has been reported in many instances in the literature [22]. Using anaerobically-treated swine waste, Miles and Ellis [23] reported a maximum of approximately 90% ammonia removal as struvite by adding external magnesium and phosphate. The optimum pH was found to be 9.5. Since struvite formation is often reported to be limited by the amount of magnesium ions available, an additional experiment was carried out by adding an external source of magnesium to the pot ale at pH 9, in order to determine whether this would cause further precipitation of ammonia. However, the results showed that the free ammonia concentration did not decrease further, therefore indicating that ammonia precipitation is not limited by the availability of magnesium ions.

3.1.5 Copper removal

Figure 6 shows the profile of digested copper as a function of pH. In general the digested copper profile is somewhat scattered, but generally, the concentration of digested copper decreases with an increase in the pH. The issue with scattered data could be linked to analytical issues arising due to the complex organic matrix of the pot ale wastewater and the strong colour of the sample which would have had a detrimental effect on the photometer reading. At the original pH a considerable difference in the raw and filtered sample readings can be observed. From filtering alone, a reduction of almost 50% of copper is achieved which suggests that a high proportion of copper in the pot ale wastewater is insoluble and bound to solids. This is agreement with other studies reported in the literature. Graham et al. [1] found total concentration of copper in pot ale to be in the range 2-5 mg/l, of which on average less than 50% was in soluble form. Quinn et al. [24] found total copper concentration in pot ale in the range 2.1-2.3 mg/l, of which a fraction varying in the range 20-70% was in soluble form. By increasing the pH to 8 or above, only a very low copper concentration was left in the filtrate, so indicating that, by a process of solid-liquid separation coupled with pH adjustment, more than 80% of the copper in pot ale can be precipitated out of the solution.  Our results are consistent with the hypothesis that copper is removed via precipitation as hydroxide. The reaction of copper hydroxide precipitation at alkaline pH can be described by the reaction below

(3)

The reaction shows that, increasing the pH, the concentration of copper in solution decreases, in agreement with our findings. However, it should be noticed that the chemistry of copper in water is very complex and many other reactions can also occur, especially in complex matrices such as pot ale.

3.2. Adsorption on chitosan

3.2.1 Effect of chitosan on pot ale pH

In the adsorption experiments it was observed that the pot ale pH increased due to the addition of chitosan. Table 2 shows the final pH after adsorption tests with the various concentration of chitosan tested, for two values of the initial pH of the pot ale. The increase in pH was dependent on the chitosan concentration and was particularly important for the tests with initial pH of the pot ale equal to 5. In this case, pH increased by almost 2 pH units (up to 6.88) at the highest chitosan concentration tested. The increase in pH was due to the protonation of the amine group on the chitosan molecule, which caused a removal of hydrogen ions, according to the following reaction [12] :

(4)

where RNH2 is the amine group on the chitosan molecule.

The increase in pH with chitosan addition had important effects on the adsorption results, as discussed in the next sections, which report the results for the experiments with initial wastewater pH equal to 5.

3.2.2 COD removal (pot ale at initial pH 5)

Figure 7 shows the residual COD in the pot ale as a function of the chitosan concentration. Organic species, measured as COD, can be removed by chitosan by two main mechanisms: electrostatic adsorption on the protonated amino group and/or binding to the hydroxyl group [25-27]. At a chitosan concentration of 10 g/l approximately 10% of the initial pot ale COD was removed. However, very little further improvement is observed for chitosan concentrations higher than 10 g/l. For the highest chitosan concentration tested, 50 g/l, a lower COD removal is observed than at 10 and 20 g/l. The initial part of the curve of the residual COD as a function of the chitosan concentration can be simply explained by the higher COD removal that is obtained by increasing the adsorbent concentration, as expected in adsorption processes. However, the fact that the residual COD did not decrease further at higher chitosan concentrations was not expected according to the standard adsorption theory and was probably due to the higher pH in the experiments at higher chitosan dosage. As shown by equation (4) in section 3.2.1 and as further discussed in section 3.2.6, a higher pH causes a lower degree of protonation for the amino groups on the chitosan molecule and this may cause a lower adsorption. The worse performance of higher chitosan concentrations on COD removal is confirmed by the adsorption isotherm, showing the adsorption capacity vs. the equilibrium COD concentration. The unusual profile of the adsorption isotherm is due to the lower COD adsorption observed at the highest chitosan concentration tested, which was linked to the higher pH. The unusual profile of the adsorption isotherm prevents a simple mathematical description of adsorption, e.g. a Langmuir or Freundlich isotherm. On the other hand, the maximum adsorption capacity, observed with the lowest chitosan concentration tested (1 g/L) was approx. 900 mg COD/g chitosan, in the same range of the adsorption capacity of chitosan for COD for other wastewaters reported in the literature. For example, Pitakpoolsil and Hansom [16] observed an adsorption capacity in the range 1000-6000 mg COD/g chitosan from a biodiesel wastewater using unmodified chitosan flakes. In a study on rice mill wastewater [15], the adsorption capacity of chitosan for COD was found to be in the range 1000-4000 mg COD/g chitosan [14,15]. Using diluted vinasse [28], chitosan adsorption capacity for COD was found to be in the range 200-500 mg COD/g chitosan. This comparison with literature studies shows that the maximum adsorption capacity of chitosan for COD in pot ale is comparable to the chitosan adsorption capacity for other effluents, therefore indicating that chitosan is potentially able to remove COD from pot ale. However, the high initial COD in pot ale and the increase in pH observed with high chitosan dosages with consequent decrease in adsorption capacity, allowed for only a modest COD removal, up to 10%, in this study.

3.2.3 Phosphorus removal (pot ale at initial pH 5)

Figure 8 shows total phosphorus and free phosphate after adsorption with chitosan at different concentrations. Phosphate removal by chitosan is usually explained on the basis of the electrostatic interactions between the negative phosphate ions and the protonated amine groups in the chitosan molecule [14,29]. To this regard, it is important to observe that in the whole pH range investigated in this study phosphate is present as negative ions, either  or . The trend of phosphate as a function of chitosan concentration is similar to what observed for COD, with a sharp decrease in adsorption capacity at chitosan concentrations larger than 10 g/l, and this can again be linked to the increase in pH. The adsorption isotherm confirms the decrease in adsorption effectiveness at higher chitosan dosages. The maximum adsorption capacity for free phosphates and for soluble phosphorus observed in this study is approximately 30 mg P/ g chitosan. The adsorption capacity of chitosan for phosphate measured in this study is in good agreement with the adsorption capacity for free phosphate, on synthetic solutions, of quaternized chitosan beads, which was found to be in the range 10-60 mg P/ g chitosan [14]. The conclusion from the results of phosphate adsorption are similar to what we observed for COD in the previous section: chitosan can be effective in removing phosphate from pot ale, however the maximum degree of phosphate removal is limited by the pH increase caused by high chitosan dosages, which decreases the adsorption capacity for phosphate. This is further discussed in section 3.2.6.

3.2.4 Ammonia and copper removal (pot ale at initial pH 5)

Ammonia was not removed at any chitosan concentration. This was probably due to the competition of other species for the active sites on chitosan. Indeed, literature studies have shown that ammonia can be removed by chitosan from synthetic solutions. Using crosslinked chitosan, an adsorption capacity of up to 120 mgN/g solid was observed [30] for synthetic solutions of ammonia in the wide pH range 4-9.

Copper was virtually completely removed at all the chitosan concentration tested, as expected on the basis of the low copper concentration in the wastewater. The observed copper removal by chitosan is in agreement with various literature studies which report the ability of chitosan to remove copper. An adsorption capacity in the range 20-80 mg Cu/g chitosan was observed for copper solutions at pH 6[31]. In another study, the maximum adsorption capacity of crosslinked chitosan for copper was 318 mg Cu/g solid [32]. The mechanism for copper removal by chitosan is generally accepted to be binding to the amino group [33].

3.2.5 Colour removal (pot ale at initial pH 5)

Figure 9 shows the effect of chitosan concentration on absorbance at four wavelengths. In general, at all the wavelengths absorbance decreases with increasing chitosan concentration, indicating that colour is removed by chitosan. Colour removal by chitosan can be explained by adsorption of organic species containing chromophore groups and/or by the removal of transition metals, which can be responsible for colour in wastewaters. Colour removal by chitosan has been shown in several literature studies [11,28,34,35].The ability of chitosan to decrease colour from pot ale is particularly important, since usually biological processes are not able to remove colour from distillery wastewaters [9]. Very few studies have investigated colour removal in pot ale, e.g. Tokuda et al.[17] investigated colour removal using coagulation with FeCl3 followed by sedimentation.

3.2.6 Experiments at different pH

The results reported in the previous section on adsorption at initial wastewater pH equal to 5 indicate only a limited degree of COD and phosphorus removal. Therefore, additional experiments were carried out to investigate whether a change in pH could determine an improvement in the adsorption capacity.

An experiment was carried out with the wastewater at initial pH 7 (the final pH was higher than 7, due to the chitosan addition, as described in section 3.2.1). No adsorption of COD, phosphorus and ammonia was observed. The results indicate that increasing pH has a negative effect on adsorption with chitosan and this is probably due to the fact that at higher pH the NH2 group on the chitosan molecule is less protonated, and therefore less effective in adsorption, than at lower pH. Indeed, chitosan adsorption is usually reported to be less effective at higher pH values due to the lower protonation of the amine group and consequent decrease in electrostatic interactions between chitosan and organic molecules [11]. In a study from biodiesel wastewater, a decrease in adsorption capacity of chitosan for COD was observed when pH increased above 4 [16]. Adsorption capacity for dyes of cross-linked chitosan was shown to decrease markedly with increasing pH [36] and, similarly, adsorption of humic acids on chitosan was shown to be more favourable at pH 6.5 than 8.5 or 12 [37]. For free phosphate, the adsorption capacity of cross-linked chitosan beads was observed to decrease with pH for pH higher than 6 [14].

Figure 10 shows a more quantitative analysis of the effect of pH on COD and free phosphate adsorption. Figure 10A, obtained combining the results of the experiments at initial pH 5 and 7, shows the adsorption capacity for COD and free phosphate as a function of the final pH of the slurry after chitosan addition. A sharp decrease in adsorption capacity with increasing pH was observed. Figure 10B shows the theoretical ratio between protonated amine groups and total amine groups on chitosan as a function of pH. The curve was calculated assuming the equilibrium reaction (4), section  3.2.1, with an equilibrium constant equal to 106.2 [12]. The curve shows that at pH 5 virtually all the amine groups are protonated, but the degree of protonation decreases sharply with increasing pH and at pH higher than 7 the degree of protonation is very low. Figure 10C combines the information from Figures 10A and 10B and shows the adsorption capacity as a function of the fraction of protonated amine groups. It is evident that for both COD and phosphate the adsorption capacity is well correlated with the fraction of protonated amine groups, showing a strong increase as the fraction of protonated groups increases. Overall, this analysis indicates the strong effect of pH on adsorption of COD and phosphate from pot ale and highlights the benefit of working at acidic pH.

In order to determine whether better adsorption performance could be obtained at lower pH, an experiment was carried out whereby the pH was controlled at the value of 6.0 by sulphuric acid addition. However, under these conditions at most of the chitosan dosages tested (10, 20 and 50 g/l) swelling of chitosan was observed and chitosan formed a thick paste with water, therefore preventing any solid-liquid separation.

The swelling of chitosan observed in the experiment at controlled pH 6.0 is in agreement with what observed in the literature. Indeed, it is generally observed that unmodified chitosan loses its integrity at acidic pH due to the protonation of the amine group, which causes partial dissolution [11]. This effect has been observed in various experimental studies [12,13].

In summary the results of our study indicate that unmodified chitosan gives a better adsorption of COD and phosphorus in pot ale when the final pH of the slurry (after chitosan addition) is in a narrow pH range, close to 6.3. At higher pH values the adsorption capacity decreases, likely due to the lower protonation of the amine group, while at more acidic pH values chitosan swelling and dissolution prevents any solid-liquid separation, especially at high chitosan concentrations. Therefore, unmodified chitosan cannot be efficiently used for COD and phosphorus removal from pot ale. The use of cross-linked chitosan, which has been shown to have better mechanical properties and better stability to acidic conditions [31], needs to be investigated.

4. Conclusions

This study shows that by adjusting pH and separating the solid and liquid fractions of pot ale the following removal of the main pollutants can be achieved: COD 14%, free phosphate 60%, total phosphorus 40%, ammonia 65%, copper >80%. Phosphorus and ammonia are only removed at pH 9, the highest investigated pH value, and this indicates that they are probably removed by precipitation. Copper is removed when pH is higher than 7, and this is also likely due to precipitation. The majority of the removed COD can be removed even at the original acidic pH of the pot ale, so indicating that a fraction of the total pot ale COD is bound to the solids.

Chitosan shows a good adsorption capacity for COD and phosphorus in pot ale, in line with other experimental studies on different wastewaters or synthetic solutions. However, the overall removal of COD and phosphorus that can be obtained with unmodified chitosan is limited to approximately 10% of COD and 35% of phosphorus. The reason for this is the decrease in adsorption capacity which is observed when pH is higher than approximately 6.3 and the instability of chitosan at pH values of 6.0 or lower. The use of chemically modified or cross-linked chitosan, with better mechanical properties and better resistance to acidic pH values, might offer a more effective treatment method for pot ale and deserves further investigation.

Acknowledgment

The authors acknowledge the financial support of the Energy Technology Partnership (ETP) under the ETP Consultancy Fund. The authors also acknowledge the skilful assistance of Mrs Liz Hendrie in performing the experiments.

References

[1] J. Graham, B. Peter, G.M. Walker, A. Wardlaw, E. Campbell, Characterisation of the pot ale profile from a malt whisky distillery, Distilled Spirits: Science and Sustainability 43 (2012),1-7.

[2] F. Jack, J. Bostock, D. Tito, B. Harrison, J. Brosnan, Electrocoagulation for the removal of copper from distillery waste streams, Journal of the Institute of Brewing 120 (2014) 60-64.

[3] G. Newbert, Energy efficient drying, evaporation and similar processes, Journal of Heat Recovery Systems 5 (1985) 551-559.

[4] J. Goodwin, J. Finlayson, E. Low, A further study of the anaerobic biotreatment of malt whisky distillery pot ale using an UASB system, Bioresource Technology 78 (2001) 155-160.

[5] P. Mallick, J.C. Akunna, G.M. Walker, Anaerobic digestion of distillery spent wash: Influence of enzymatic pre-treatment of intact yeast cells, Bioresource Technology 101 (2010) 1681-1685.

[6] N. Uzal, C.F. Gökçay, G.N. Demirer, Sequential (anaerobic/aerobic) biological treatment of malt whisky wastewater, Process Biochemistry 39 (2003) 279-286.

[7] S. Mohana, B.K. Acharya, D. Madamwar, Distillery spent wash: treatment technologies and potential applications, Journal of Hazardous Materials 163 (2009) 12-25.

[8] P.J. Strong, J.E. Burgess, Treatment methods for wine-related and distillery wastewaters: a review, Bioremediation Journal 12 (2008) 70-87.

[9] Y. Satyawali, M. Balakrishnan, Wastewater treatment in molasses-based alcohol distilleries for COD and color removal: a review, Journal of Environmental Management 86 (2008) 481-497.

[10] G. Crini, Recent developments in polysaccharide-based materials used as adsorbents in wastewater treatment, Progress in Polymer Science 30 (2005) 38-70.

[11] G. Crini, P. Badot, Application of chitosan, a natural aminopolysaccharide, for dye removal from aqueous solutions by adsorption processes using batch studies: A review of recent literature, Progress in Polymer Science 33 (2008) 399-447.

[12] R. Juang, H. Shao, Effect of pH on competitive adsorption of Cu (II), Ni (II), and Zn (II) from water onto chitosan beads, Adsorption 8 (2002) 71-78.

[13]M. Chiou, H. Li, Adsorption behavior of reactive dye in aqueous solution on chemical cross-linked chitosan beads, Chemosphere 50 (2003) 1095-1105.

[14] A. Sowmya, S. Meenakshi, An efficient and regenerable quaternary amine modified chitosan beads for the removal of nitrate and phosphate anions, Journal of Environmental Chemical Engineering 1 (2013) 906-915.

[15] K. Thirugnanasambandham, V. Sivakumar, J. Prakash Maran, Application of chitosan as an adsorbent to treat rice mill wastewater—Mechanism, modelling and optimization, Carbohydrate Polymers 97 (2013) 451-457.

[16] W. Pitakpoolsil, M. Hunsom, Adsorption of pollutants from biodiesel wastewater using chitosan flakes, Journal of the Taiwan Institute of Chemical Engineers 44 (2013) 963-971.

[17] M. Tokuda, Y. Fujiwara, K. Kida, Pilot plant test for removal of organic matter, N and P from whisky pot ale, Process Biochemistry 35(1999) 267-275.

[18] C. Jackson, G. Lines, Measures against water pollution in the fermentation industries, Pure and Applied Chemistry 29 (1972) 381-394.

[19] P. Koutsoukos, Z. Amjad, M. Tomson, G. Nancollas, Crystallization of calcium phosphates. A constant composition study, Journal of the American Chemical Society 102 (1980) 1553-1557.

[20] Y. Song, H.H. Hahn, E. Hoffmann, Effects of solution conditions on the precipitation of phosphate for recovery: A thermodynamic evaluation, Chemosphere 48 (2002) 1029-1034.

[21] J.D. Doyle, S.A. Parsons, Struvite formation, control and recovery, Water Research 36 (2002) 3925-3940.

[22] N. Booker, A. Priestley, I. Fraser, Struvite formation in wastewater treatment plants: opportunities for nutrient recovery, Environmental Technology 20 (1999) 777-782.

[23] A. Miles, T.G. Ellis, Struvite precipitation potential for nutrient recovery from anaerobically treated wastes, Water Science & Technology 43 (2001) 259-266.

[24] J. Quinn, T. Barker, R. Marchant, Relationship between free and complexed forms of copper in malt whiskey distillery spent wash, Journal of the Institute of Brewing 86 (1980) 274-276.

[25] Y. Wong, Y. Szeto, W. Cheung, G. McKay, Equilibrium studies for acid dye adsorption onto chitosan, Langmuir 19 (2003) 7888-7894.

[26] T.K. Saha, S. Karmaker, H. Ichikawa, Y. Fukumori, Mechanisms and kinetics of trisodium 2-hydroxy-1, 1′-azonaphthalene-3, 4′,6-trisulfonate adsorption onto chitosan, Journal of Colloid and Interface Science 286 (2005) 433-439.

[27] G. Dotto, L. Pinto, Adsorption of food dyes acid blue 9 and food yellow 3 onto chitosan: Stirring rate effect in kinetics and mechanism, Journal of Hazardous Materials 187 (2011) 164-170.

[28] I.G. Lalov, I.I. Guerginov, M.A. Krysteva, K. Fartsov, Treatment of waste water from distilleries with chitosan, Water Research 34 (2000) 1503-1506.

[29] R.M. Nthumbi, J. Catherine Ngila, B. Moodley, A. Kindness, L. Petrik, Application of chitosan/polyacrylamide nanofibres for removal of chromate and phosphate in water, Physics and Chemistry of the Earth, Parts A/B/C 50 (2012) 243-251.

[30] Y. Zheng, A. Wang, Evaluation of ammonium removal using a chitosan-g-poly (acrylic acid)/rectorite hydrogel composite, Journal of Hazardous Materials 171 (2009) 671-677.

[31] W. Wan Ngah, C. Endud, R. Mayanar, Removal of copper (II) ions from aqueous solution onto chitosan and cross-linked chitosan beads, Reactive and Functional Polymers 50 (2002) 181-190.

[32] G.Z. Kyzas, M. Kostoglou, N.K. Lazaridis, Copper and chromium (VI) removal by chitosan derivatives—Equilibrium and kinetic studies, Chemical Engineering Journal 152 (2009) 440-448.

[33] C. Gerente, V. Lee, P.L. Cloirec, G. McKay, Application of chitosan for the removal of metals from wastewaters by adsorption—mechanisms and models review, Critical Reviews in Environmental Science and Technology 37(2007) 41-127.

[34] R. Juang, R. Tseng, F. Wu, S. Lin, Use of chitin and chitosan in lobster shell wastes for color removal from aqueous solutions, Journal of Environmental Science & Health Part A 31(1996) 325-338.

[35] M.N. Ravi Kumar, A review of chitin and chitosan applications, Reactive and Functional Polymers 46 (2000) 1-27.

[36] M. Chiou, P. Ho, H. Li, Adsorption of anionic dyes in acid solutions using chemically cross-linked chitosan beads, Dyes and Pigments 60 (2004) 69-84.

[37] X. Zhang, R. Bai, Mechanisms and kinetics of humic acid adsorption onto chitosan-coated granules, Journal of Colloid and Interface Science 264 (2003) 30-38.

Table 1. Composition of the pot ale used in this study

| Parameter | Value | Units |
| --- | --- | --- |
| pH | 3.4 |  |
| TSS | 5.3 | g/l |
| Raw COD | 83.4 | g/l |
| Filtered COD | 75.9 | g/l |
| Filtered ammonia | 0.3 | gN/l |
| Raw digested phosphorus | 0.7 | gP/l |
| Filtered digested phosphorus | 0.7 | gP/l |
| Filtered free phosphate | 0.5 | gP/l |
| Raw digested copper | 1.3 | mg/l |
| Filtered digested copper | 0.3 | mg/l |

Table 2. Effect of chitosan concentration on the final pH, for two values of the initial pot ale pH.

| Chitosan dose (g/l) | Final pH for the wastewater at initial pH 5.03 | Final pH for the wastewater at initial pH 7.06 |
| --- | --- | --- |
| 0 | 4.98 | 7.12 |
| 1 | 5.26 | 7.29 |
| 10 | 6.27 | 7.24 |
| 20 | 6.50 | 7.33 |
| 50 | 6.88 | 7.46 |

Barley

Draff

Whisky

Maturation

Second distillation (Spirit still)

Distillation

Fermentation

Mashing

Malting

Water

Malt

Wort

Yeast

Wash

Pot ale

Low wines

Spent lees

Spirit

Figure 1. Main stages in the whisky production process. Pot ale, target of this study, is circled.

Figure 2. Base concentration required to increase pH of pot ale.

Figure 3. Raw pot ale COD and filtered values as a function of pH.

Figure 4. Raw phosphorus, filtered digested phosphorus and free phosphate as a function of pH.

Figure 5. Free ammonia as a function of pH.

Figure 6. Copper in raw and filtered pot ale as a function of pH.

Figure 7. A) Residual COD after adsorption with chitosan at various concentrations of chitosan. B) adsorption isotherm. Experiments with initial wastewater at pH 5.

Figure 8. A) Residual phosphorus and phosphate after adsorption with chitosan at various concentrations of chitosan. B) adsorption isotherm. Experiments with initial wastewater at pH 5.

Figure 9. Colour removal after adsorption with chitosan. Experiments with initial wastewater at pH 5.

Figure 10. Effect of pH on adsorption. A) Adsorption capacity as a function of final pH for COD and phosphorus; B) Theoretical fraction of protonated amine groups on chitosan as a function of pH; C) Adsorption capacity for COD and phosphorus as a function of the fraction of protonated amine groups on chitosan.